Autonomous 3D Capture Drone System

Complete Project Reference & Development History

Hard (Siddhesh Darak)

Cornell University - Graduate Research

Last Updated: March 2026

Hardware: DJI Mini 4 Pro | RC-N3 Controller | Android MSDK V5

Development Laptop: "antigravity"

# 1. The Vision

Build an end-to-end autonomous drone system for one-click 3D Gaussian splat reconstruction. The user places an object, the drone autonomously captures it from all angles, and a pipeline outputs a polished 3D model. No manual flying. No manual photogrammetry. Press a button, get a 3D model.

### Complete Pipeline

- User places object on surface

- Drone takes off and flies over object to measure height (ToF/VPS differential altimetry)

- Indoor positioning system tracks drone location in real-time (no GPS)

- Drone executes multi-altitude dome capture pattern around object

- Real-time object recognition identifies and tracks target

- Gaussian splatting pipeline generates 3D reconstruction

- User receives polished 3D model

### Project Status Overview

| Component | Status | Notes |
| --- | --- | --- |
| Orbital flight patterns (KMZ generator) | BUILT | Python-based, 58-waypoint dome, 4 altitude levels |
| WPML reverse-engineering | BUILT | DJI waypoint format decoded, KMZ generation working |
| SD card mission deployment | BUILT | Bypass RC 2 limitations via drone SD card |
| Outdoor GPS flight test | READY | Coordinates set: 40.754972, -73.956167 |
| Indoor positioning system | R&D | Evaluating Marvelmind, ArUco, UWB, SLAM |
| MSDK V5 Android app | IN PROGRESS | Environment setup on antigravity laptop |
| Object height measurement | CONCEPT | ToF differential altimetry validated theoretically |
| Object recognition | PLANNED | Real-time target identification |
| Gaussian splat generation | PLANNED | Cornell research with professor |
| User application | PLANNED | Seamless end-to-end interface |

# 2. Hardware Setup & Evolution

## 2.1 Current Hardware

Drone: DJI Mini 4 Pro

Controller: RC-N3 (acquired Feb 2026)

Development Device: Android phone (borrowed, needs Android 8.0+)

Development Laptop: "antigravity"

SDK: DJI Mobile SDK V5

## 2.2 Controller Evolution

The project went through three controller stages, each unlocking new capabilities:

| Controller | Capabilities | Limitations |
| --- | --- | --- |
| RC 2 (original) | DJI Fly app, basic waypoints, SD card file transfer workaround | Locked-down Android, no MSDK, no ADB, no third-party apps |
| RC-N2 (researched) | MSDK app support, Virtual Stick API, phone-based development, third-party apps (Litchi, Dronelink) | Still requires custom app for indoor flight |
| RC-N3 (current) | Full MSDK V5 support, real-time waypoint missions, flight telemetry access, programmatic gimbal control, USB connection to Android device | Requires separate Android device |

Key insight: The RC 2 was designed for casual users. The RC-N3 unlocks everything needed for autonomous development.

## 2.3 Drone Sensor Array

The DJI Mini 4 Pro has extensive sensors, but accessibility varies:

- Main gimbal camera - Accessible via MSDK, used for capture AND potentially navigation

- Downward binocular vision cameras - Firmware-locked, NOT accessible via MSDK

- Fisheye cameras - Firmware-locked, NOT accessible via MSDK

- 3D Infrared Time-of-Flight (ToF) sensor - Used internally for VPS, not directly accessible

- Auxiliary bottom lighting - Automatic, not controllable

Critical limitation: Only the main gimbal camera feed is accessible through Mobile SDK. All VPS cameras are firmware-locked.

# 3. What Was Built: Python Waypoint Mission Generator

## 3.1 WPML Reverse Engineering

DJI's waypoint missions use the WPML format, which is essentially a KMZ file (ZIP archive) containing XML files. This format was reverse-engineered from DJI's Cloud API documentation and practical experimentation.

### WPML File Structure

- KMZ file (ZIP format) containing template.kml and waylines.wpml

- template.kml defines mission metadata, drone model, payload settings

- waylines.wpml contains the actual waypoint coordinates and actions

- Supports waypoint types: fly-to-point, curved flight, hover

- Actions at waypoints: take photo, start/stop recording, gimbal rotation

## 3.2 Dome Capture Pattern

The Python generator creates multi-altitude orbital dome capture patterns optimized for 3D reconstruction:

| Parameter | Value | Purpose |
| --- | --- | --- |
| Total waypoints | 58 | Full dome coverage |
| Altitude levels | 4 (0.9m, 1.7m, 2.7m, 3.3m) | Multi-elevation capture |
| Elevation bands | 25°, 50°, 75° | Hemisphere coverage |
| Points per band | 10, 8, 5 (lower = more) | Denser at lower angles |
| Dome radius | 3.0m (adjustable) | Distance from object |
| Flight speed | <1 m/s precision | Smooth capture |
| POI tracking | 360° continuous | Camera always faces object |
| Gimbal angles | -25°, -50°, -75° | Progressive downward tilt |

## 3.3 Key Technical Features

- Continuous POI tracking - Drone heading always faces the center object at every waypoint

- Smooth curved flight paths - Uses WPML curved turn mode for cinematographic quality

- Gimbal automation - Gimbal pitch calculated per waypoint based on elevation angle to object center

- Video recording automation - Start/stop recording actions embedded in waypoint actions

- GPS coordinate generation - Converts dome geometry into lat/lon using distance-bearing calculations

- Configurable parameters - Object height, dome radius, number of bands, points per band all adjustable

## 3.4 SD Card Deployment Workaround

With the RC 2 controller (no MSDK access), a workaround was discovered:

- Create a dummy waypoint mission in DJI Fly (requires outdoor GPS lock)

- Access the drone's SD card via the RC 2 file manager

- Navigate to the waypoint folder (random hex-named directories)

- Replace the .kmz file with the custom-generated dome capture mission

- Load mission in DJI Fly and execute

This method was validated but has limitations: GPS waypoints don't work indoors, and the workaround is fragile.

## 3.5 Test Coordinates

Location: 40°45'17.9"N 73°57'22.2"W

Decimal: 40.754972, -73.956167

Takeoff: 2m south of object center

# 4. Indoor Positioning Research

GPS-based waypoint missions do not work indoors. This is a hard limit in DJI's MSDK - the WaypointMission class requires GPS lock to initialize. All indoor navigation must use the Virtual Stick API with an external positioning system.

## 4.1 The Core Problem

The DJI Mini 4 Pro's Vision Positioning System (VPS) handles hover stability effectively - the drone can hold position indoors. However, VPS does NOT provide absolute room coordinates. It only maintains relative position from its current hover point. Over time, drift accumulates, affecting multi-orbit coverage patterns.

There is also a fundamental conflict: the main gimbal camera is the only camera accessible via MSDK. If you point it at positioning markers for navigation, you can't simultaneously point it at the object for capture.

## 4.2 Positioning Alternatives Evaluated

### Option 1: Marvelmind Indoor GPS (Recommended)

- Ultrasonic beacon-based positioning, ±2cm accuracy

- Proven working with DJI Mini 4 Pro + RC-N2 setup

- Camera-independent - no conflict with capture gimbal

- Requires 4+ stationary beacons + 1 mobile beacon on drone

- Cost: $500-800 for starter kit + software license

- Android app streams position via UDP, controls drone via Virtual Stick

### Option 2: ArUco Markers

- Printed fiducial markers detected via OpenCV

- Free to produce, well-documented library support

- Camera must see them - conflicts with gimbal pointing at subject

- Accuracy degrades beyond 3-4m, affected by lighting

- Possible interleaved workaround: gimbal down (detect) → fly segment → gimbal to subject (capture) → repeat

- Works for still photos only, not continuous video

### Option 3: AprilTags

- Better-engineered alternative to ArUco

- Lower false detection rates, more robust at oblique angles

- Same fundamental camera conflict limitation

### Option 4: UWB (Ultra-Wideband) Systems (e.g., Pozyx)

- Camera-independent positioning

- Less precise than Marvelmind for this application

- Requires anchor infrastructure around the room

### Option 5: Visual SLAM on Main Camera

- Free, no additional hardware needed

- Complex to implement, conflicts with capture (camera shared)

- Heavy computation - best offloaded to nearby laptop

### Option 6: Ceiling-Mounted Camera Tracking

- External camera(s) mounted above track the drone from outside

- Camera-independent on the drone side

- Leverages existing Python/CV skills

- Requires room infrastructure setup

### Option 7: Colored/Patterned Floor Mats

- Large printed mat with unique gradient or pattern

- CV determines position by recognizing visible section

- Same camera conflict as marker-based approaches

## 4.3 Key Technical Insight

For the orbital capture use case, camera-independent solutions (Marvelmind, UWB, ceiling cameras) are strongly preferred because they allow the gimbal to focus entirely on the capture subject while the drone navigates autonomously.

# 5. MSDK V5 Development Setup

## 5.1 Architecture

With the RC-N3 controller, the system architecture is:

- Android phone connects to RC-N3 via USB

- Custom Android app runs DJI Mobile SDK V5

- App sends Virtual Stick commands for indoor flight control

- App receives live video feed via VideoFeeder API

- External positioning data fed into app (Marvelmind/UWB/etc.)

- App manages gimbal orientation and camera capture

## 5.2 MSDK Indoor Flight: Virtual Stick API

Since WaypointMission requires GPS, all indoor flight uses Virtual Stick Control Mode:

- Send FlightControlData: pitch, roll, yaw, and vertical throttle

- Implement closed-loop feedback using external position data

- PID controller needed to handle latency in the control chain

- Control chain latency: Drone → Remote → Phone → Server → Phone → Remote → Drone

## 5.3 Video Streaming for External Processing

MSDK provides access to the live video stream via the VideoFeeder API:

- Raw YUV or H.264 frames accessible on the Android device

- Can stream via TCP/UDP/WebRTC to external compute (laptop/server)

- For lightweight drones like Mini 4 Pro, heavy processing (SLAM, object detection) is better offloaded to nearby laptop

- Local processing possible with OpenCV/TFLite for low-latency tasks

## 5.4 Development Environment (In Progress)

Laptop: "antigravity"

IDE: Android Studio

JDK: JDK 11+

Android SDK: API Level 31+ (for MSDK V5)

MSDK Version: DJI Mobile SDK V5 (Android-only)

Test Device: Android phone with Android 8.0+ (10+ recommended)

### Gradle Dependencies (MSDK V5)

- DJI aircraft SDK V5 package

- Kotlin/Java support

- USB Host mode for controller connection

### Setup Steps Completed/In Progress

- Android Studio installation on antigravity

- SDK Manager configuration (API 31+)

- JDK setup and verification

- DJI Developer Account registration + App Key

- USB debugging enabled on test Android device

- MSDK V5 Gradle dependency integration (next step)

- Project structure and MSDK initialization code (next step)

# 6. Object Height Measurement Concept

Before executing the dome capture pattern, the drone needs to know the object's height to calculate optimal orbit altitudes and gimbal angles.

## 6.1 Differential Altimetry Approach

The concept uses the drone's VPS sensors (ToF + binocular vision) to measure height:

- Drone hovers over the floor next to the object → measures distance to floor (h_floor)

- Drone moves over the object → measures distance to object top (h_object)

- Object height = h_floor - h_object (at same drone altitude)

ToF Formula: h = c·t/2 (speed of light × photon round-trip time / 2)

Target Accuracy: Sub-decimeter in GPS-denied environments

## 6.2 Feasibility Assessment

- VPS sensors perform these calculations internally for hover stability

- RC 2 doesn't expose raw sensor data; MSDK with RC-N3 may provide altitude readings

- Practical workaround: Use displayed altitude readings for manual-assisted measurement

- Concept is sound for single-altitude orbits at close range

- Multi-altitude missions need position correction to counter VPS drift over time

# 7. MSDK Technical Analysis (from RIIS LLC Research)

Based on analysis of RIIS LLC's deep dives into DJI Mobile SDK, these are the key findings for indoor autonomous flight:

## 7.1 Three Core Questions Answered

### Q1: Is vision-based positioning reliable for indoor flight paths?

- VPS is stable for hovering and short-distance maneuvers

- Drift is inevitable over long paths - no true indoor GPS

- Closed-loop feedback system required using video feed or external sensors

- Virtual Stick Control Mode provides FlightControlData (pitch, roll, yaw, vertical throttle)

### Q2: Can camera feed stream to external compute in real-time?

- Yes - core MSDK capability via VideoFeeder API

- Socket streaming (TCP/UDP/WebRTC) from mobile app to external server

- For lighter drones, offload heavy computation (SLAM, detection) to nearby laptop

- On-device processing possible with OpenCV or TFLite for low-latency needs

### Q3: Hard limitations for indoor autonomous navigation?

- No waypoint missions indoors - WaypointMission class requires GPS lock - hard limit. Must use Virtual Stick API exclusively

- Network latency - Full round-trip chain (drone→remote→phone→server→back) causes oscillations. PID tuning critical

- MSDK V4 vs V5 incompatibility - V5 has completely different architecture. V5 is Android-only with different streaming APIs. Must match SDK version to hardware

# 8. Drift & Position Correction Strategies

VPS drift accumulation is a key challenge for multi-orbit dome capture missions. Several strategies were discussed:

- Return to a known position between altitude layers to reset drift reference

- Use visual markers on the ground for continuous position correction

- Accept some imperfection and rely on photogrammetry software (Meshroom, RealityCapture) to handle slight overlap inconsistencies - these tools are surprisingly forgiving

- For single-altitude orbits at close range, drift is minimal and acceptable

- For full dome coverage (multiple altitudes), active correction becomes important

- Marvelmind or UWB provides continuous absolute position feedback independent of drift

# 9. Research Context & Professional Outreach

## 9.1 Cornell Research

Working with a professor at Cornell on autonomous drone systems research, specifically focused on creating a Gaussian splatting pipeline for 3D reconstruction using autonomous drone image capture. The research involves mapping real-time drone captures against architectural CAD models to eliminate positioning drift in GPS-denied environments.

## 9.2 Background & Experience

- President of university's Unmanned Aerial Systems (UAS) Group during undergrad

- Built competition drones and competed in multiple events

- Hands-on experience with drone hardware: Pixhawk flight controllers, PX4, ArduPilot, Betaflight

- Drone software protocols: MAVSDK, MAVLink, QGroundControl, Mission Planner, DroneKit

- Computer vision: YOLOv8, YOLOv5s, OpenCV, pose estimation, object detection

- Related projects: Human Anomaly Detection System (YOLOv8 + MobileLSTM, 90%/97% accuracy), Automated Tree Counting for Agriculture using UAVs (YOLOv5s, 95% accuracy across 500+ farmland)

- Publication: "American Sign Language Recognition and Detection: A Review" - ICIVC (2023)

## 9.3 Theseus Outreach (Defense Tech)

Crafted cold outreach email to Theseus (YC S24 defense tech startup) building GPS-denied navigation for drones using visual odometry + satellite map matching. Strong technical overlap with this project's indoor positioning challenges.

Contacts: Ian (ian@theseus.us), Sacha (sacha@theseus.us)

Key angle: Cornell research on Gaussian splatting + positioning systems + drone hardware experience

Resume: Restructured to emphasize drone systems, GPS-denied navigation, positioning research, open-source drone ecosystem (MAVLink, PX4, ArduPilot, QGC)

## 9.4 DJI Developer Expert Outreach

Drafted email to a DJI MSDK expert found via RIIS LLC YouTube content, seeking validation of whether MSDK can support indoor autonomous flight requirements with the Mini 4 Pro + RC-N3 setup.

# 10. Resume Descriptions (STAR Format)

## 10.1 Full STAR Description

SITUATION: Consumer drones lack integrated solutions for automated 3D object scanning, requiring manual flight and separate photogrammetry workflows. Indoor environments present additional challenges due to GPS unavailability.

TASK: Design and develop an end-to-end autonomous drone system that captures objects from multiple angles indoors and generates 3D reconstructions (Gaussian splats) with minimal user intervention.

ACTION: Reverse-engineered DJI's WPML waypoint format to build a Python-based orbital flight path generator producing smooth, multi-altitude dome capture patterns. Developed gimbal control algorithms ensuring continuous POI tracking across varying elevations (0°-75°) and distances. Implemented SD card-based mission deployment workflow bypassing RC 2 controller limitations. Researching SLAM-based indoor positioning to enable GPS-free autonomous navigation via MSDK. Architecting real-time video pipeline for onboard/offboard object recognition integration.

RESULT: Working prototype generates optimized flight missions for DJI Mini 4 Pro capturing 58-point orbital video sequences. System designed to scale into full autonomous 3D scanning product.

## 10.2 Dense 2-Line Version

Developing end-to-end autonomous drone system for one-click 3D Gaussian splat reconstruction—user places object, drone captures, pipeline outputs model. Built Python-based orbital flight generator reverse-engineering DJI's WPML format: 58-waypoint dome patterns across 4 altitude levels (0.9m-3.3m), 360° continuous POI tracking, <1m/s precision flight. Implementing SLAM-based indoor positioning for GPS-free navigation, MSDK real-time flight control, and onboard object recognition for fully autonomous capture pipeline.

Technologies: Python, DJI WPML/KMZ, MSDK, SLAM, OpenCV, Gaussian Splatting

# 11. Complete Technical Skills Reference

| Category | Skills |
| --- | --- |
| Drone Hardware & Firmware | Pixhawk flight controllers, PX4, ArduPilot, Betaflight, DJI Mobile SDK (MSDK V5), WPML format, Hardware integration, Custom drone builds |
| Drone Software & Protocols | MAVSDK, MAVLink protocol, QGroundControl (QGC), Mission Planner, DroneKit, Autonomous waypoint missions, Visual Positioning Systems (VPS) |
| Positioning & Navigation | GPS-denied navigation, ArUco markers, UWB positioning, Visual SLAM, Sensor fusion, IMU integration, Real-time localization, Marvelmind beacons |
| Computer Vision & ML | YOLOv8, YOLOv5s, PyTorch, TensorFlow, OpenCV, Object detection, Pose estimation, 3D reconstruction, Gaussian Splatting |
| Languages | Python, C++, JavaScript |
| ML/AI Frameworks | PyTorch, TensorFlow, scikit-learn, Keras, JAX, RNNs, Transformers |
| Data & Statistics | NumPy, Pandas, Matplotlib, Seaborn, SciPy, Jupyter |
| DevOps & MLOps | Docker, Kubernetes, Kubeflow, MLflow, Terraform, Jenkins, AWS EMR, CI/CD |
| Databases | MySQL, PostgreSQL |

# 12. Next Steps & Roadmap

## 12.1 Immediate (Environment Setup)

- Complete Android Studio setup on antigravity laptop

- Configure MSDK V5 Gradle dependencies

- Register DJI Developer Account and obtain App Key

- Build and deploy DJI Sample App to test device

- Verify RC-N3 → Android device → MSDK connection

## 12.2 Short-Term (MSDK App Development)

- Build custom Android app with Virtual Stick control

- Implement live video feed access via VideoFeeder

- Test basic indoor flight control (hover, move, rotate)

- Validate indoor flight feasibility without GPS

- Test gimbal programmatic control for POI tracking

## 12.3 Medium-Term (Indoor Positioning Integration)

- Select and procure positioning system (Marvelmind recommended, or build ArUco/SLAM solution)

- Integrate position data feed into MSDK app

- Implement closed-loop position control with PID tuning

- Test single-altitude indoor orbit around object

- Implement object height measurement via altitude differential

## 12.4 Long-Term (Full Pipeline)

- Multi-altitude dome capture missions indoors

- Real-time object recognition and tracking

- Gaussian splatting pipeline integration

- End-to-end user application (place object → get 3D model)

- Drift correction and mission optimization

- Position correction strategies for extended operations

## 12.5 Professional Development

- Follow up with Theseus (defense tech) if outreach email sent

- Connect with DJI MSDK expert for technical validation

- Continue Cornell research collaboration on Gaussian splatting

- Explore additional defense/drone industry networking opportunities

# 13. Key Learnings & Technical Decisions Log

- DJI's VPS handles hover stability but does NOT provide absolute room coordinates needed for autonomous navigation

- VPS drift accumulates over time, affecting multi-orbit coverage - shorter missions are more reliable

- Only the main gimbal camera is accessible via MSDK - VPS cameras are firmware-locked

- GPS-based waypoint missions (WaypointMission class) will NOT work indoors - hard limit

- Virtual Stick API is the only path for indoor autonomous control

- Camera conflict: navigation markers vs. capture subject - camera-independent positioning is preferred

- MSDK V4 and V5 are architecturally incompatible - V5 is Android-only

- RC 2 is locked down but SD card file replacement provides limited workaround

- RC-N3 with phone-based development unlocks full MSDK capabilities

- PID controller tuning is critical due to latency in the drone→controller→phone→server control chain

- Photogrammetry software (Meshroom, RealityCapture) is forgiving of slight position errors

- Marvelmind has proven integration with Mini 4 Pro + RC-N2/N3 - most battle-tested path

- ToF differential altimetry concept is sound but sensor data access depends on MSDK exposure

- DJI Mini 4 Pro was only added to MSDK in March 2025 - documentation may be limited

# 14. Important Links & Resources

- DJI Developer Portal - https://developer.dji.com

- DJI Mobile SDK V5 - https://developer.dji.com/mobile-sdk/

- DJI WPML Reference - https://developer.dji.com/doc/cloud-api-tutorial/en/api-reference/dji-wpml/template-kml.html

- RIIS LLC (MSDK tutorials) - YouTube channel with DJI SDK deep dives

- Marvelmind - https://marvelmind.com - Indoor positioning system

- Theseus - https://theseus.us - GPS-denied navigation startup (YC S24)

- OpenMTP - https://openmtp.ganeshrvel.com - Android file transfer for Mac

- Dronelink / Litchi - Third-party apps for DJI drone control (orbit/dome features)